1926.602(c)

Lifting and hauling equipment (other than equipment covered under Subpart N of this part).

1926.602(c)(1)

Industrial trucks shall meet the requirements of 1926.600 and the following:

1926.602(c)(1)(i)

Lift trucks, stackers, etc., shall have the rated capacity clearly posted on the vehicle so as to be clearly visible to the operator. When auxiliary removable counterweights are provided by the manufacturer, corresponding alternate rated capacities also shall be clearly shown on the vehicle. These ratings shall not be exceeded.

1926.602(c)(1)(ii)

No modifications or additions which affect the capacity or safe operation of the equipment shall be made without the manufacturer's written approval. If such modifications or changes are made, the capacity, operation, and maintenance instruction plates, tags, or decals shall be changed accordingly. In no case shall the original safety factor of the equipment be reduced.

1926.602(c)(1)(iii)

If a load is lifted by two or more trucks working in unison, the proportion of the total load carried by any one truck shall not exceed its capacity.

..1926.602(c)(1)(iv) 

1926.602(c)(1)(iv)

Steering or spinner knobs shall not be attached to the steering wheel unless the steering mechanism is of a type that prevents road reactions from causing the steering handwheel to spin. The steering knob shall be mounted within the periphery of the wheel.

1926.602(c)(1)(v)

All high lift rider industrial trucks shall be equipped with overhead guards which meet the configuration and structural requirements as defined in paragraph 421 of American National Standards Institute B56.1-1969, Safety Standards for Powered Industrial Trucks.

1926.602(c)(1)(vi)

All industrial trucks in use shall meet the applicable requirements of design, construction, stability, inspection, testing, maintenance, and operation, as defined in American National Standards Institute B56.1-1969, Safety Standards for Powered Industrial Trucks.

1926.602(c)(1)(vii)

Unauthorized personnel shall not be permitted to ride on powered industrial trucks. A safe place to ride shall be provided where riding of trucks is authorized.

1926.602(c)(1)(viii)

Whenever a truck is equipped with vertical only, or vertical and horizontal controls elevatable with the lifting carriage or forks for lifting personnel, the following additional precautions shall be taken for the protection of personnel being elevated.

1926.602(c)(1)(viii)(A)

Use of a safety platform firmly secured to the lifting carriage and/or forks.

..1926.602(c)(1)(viii)(B) 

1926.602(c)(1)(viii)(B)

Means shall be provided whereby personnel on the platform can shut off power to the truck.

1926.602(c)(1)(viii)(C)

Such protection from falling objects as indicated necessary by the operating conditions shall be provided.

1926.602(d)

Powered industrial truck operator training. 

Note: The requirements applicable to construction work under this paragraph are identical to those set forth at §1910.178(l) of this chapter.

[44 FR 8577, Feb. 9, 1979; 44 FR 20940, Apr. 6, 1979, as amended at 58 FR 35183, June 30, 1993; 63 FR 66274, Dec. 1, 1998]